Datalogging

# Adafruit Precision NXP 9-DOF Breakout Board - FXOS8700 + FXAS21002

# https://www.adafruit.com/product/3463?gclid=Cj0KCQjwhqaVBhCxARIsAHK1tiPm_dugqNACA1qpoKYiIUcRkEzozLwqOpy1czVeEUHo_wplAhaXH-EaApL6EALw_wcB

# $14.95

## Technical Details

The NXP Precision 9DoF board consists of two separate ICs, described in detail below:

FXOS8700 3-Axis Accelerometer/Magnetometer

- 2-3.6V Supply

- ±2 g/±4 g/±8 g adjustable acceleration range

- ±1200 µT magnetic sensor range

- Output data rates (ODR) from 1.563 Hz to 800 Hz

- 14-bit ADC resolution for acceleration measurements

- 16-bit ADC resolution for magnetic measurements

FXAS21002 3-Axis Gyroscope

- 2-3.6V Supply

- ±250/500/1000/2000°/s configurable range

- Output Data Rates (ODR) from 12.5 to 800 Hz

- 16-bit digital output resolution

- 192 bytes FIFO buffer (32 X/Y/Z samples)

Benefits: accel(x,y,z), gyro(x,y,z), mag(x,y,z), Ability to increase accuracy of acceleration with accelerometer on cpb

SUCCESFULL LOG ON CPB AND BLUETOOTH UART

Format: (time, gyro (x,y,z), mag(x,y,z), accel(x,y,z), temp)

Raw Code:

https://github.com/TheJus10Dyer/10/blob/main/Aerospace/NPX9DOFCPBlogger.py

Raw Data:: GYRO,MAG,ACCEL from IMU, Temp and Time from CPB

https://github.com/TheJus10Dyer/10/blob/main/Aerospace/CPB_Datalog_NXP9DOF_test.txt

Strapped on bike saved to circuitplayground

Plot

Was included in launch test more to come

Want to test on airplane to see if ideal for model planes

DUAL I2C Issues:::

Not sure if this is circuitplayground or hardware knowledge issue but could not address two separate sensors via I2C on SDA and SCL pins; therefor, could not experiment with multiple sensors that require I2C

Since dual I2C was not an option, decided to test all sensors individually.

# Adafruit LSM6DS33 + LIS3MDL - 9 DoF IMU with Accel / Gyro / Mag - STEMMA QT Qwiic

https://www.adafruit.com/product/4485

$19.95

## Technical Details

LSM6DS33 Specifications:

- ±2/±4/±8/±16 g full scale

- ±125/±250/±500/±1000/±2000 dps full scale

- Up to 1.6 KHz ODR

- Tap and double-tap detection

- Free-fall detection

- I2C Address 0x6A or 0x6B

LIS3MDL Specifications:

- ±4/±8/±12/±16 gauss selectable magnetic full scales

- Continuous and single-conversion modes

- 16-bit data output

- Interrupt generator

- I2C Address 0x1D or 0x1E

Product Dimensions: 25.6mm x 17.8mm x 4.6mm / 1.0" x 0.7" x 0.2"

Benefits: accel(x,y,z), gyro(x,y,z), mag(x,y,z), Ability to increase accuracy of acceleration with accelerometer on cpb

FAIL: MAGNETOMETER MALFUNCTIONS

SUCCESFULL LOG ON CPB AND BLUETOOTH UART

Format: (time, gyro (x,y,z), mag(x,y,z), accel(x,y,z), temp)

Raw Code:

https://github.com/cmontalvo251/Microcontrollers/blob/bd06a7afe87558c23160229cca8c2dbeac980799/Circuit_Playground/CircuitPython/CPB_DataLogger/bluefruit_datalogger_LSMLIS_uart_send.py Raw Data::

Bike test

Need data and format from C

Need Plots

Setup:

# Adafruit MS8607 Pressure Humidity Temperature PHT Sensor - STEMMA QT / Qwiic

https://www.adafruit.com/product/4716

$14.95

## Technical Details

SPECIFICATIONS

- Integrated pressure, humidity and temperature sensor

- QFN package 5 x 3 x 1 mm3

- Operating range: 10 to 2000 mbar, 0%RH to 100%RH, -40 to 85 °C

- High-resolution module: 0.016 mbar, 0.04%RH, 0.01°C

- Fully factory calibrated sensor

- I2C interface

Benefits: Pressure, relative humidity, and temperature. Ability to increase accuracy of temperature with thermistor on cpb

SUCCESFULL LOG ON CPB AND BLUETOOTH UART

Format:

Raw Code:

https://github.com/cmontalvo251/Microcontrollers/blob/bbb17ac0a4af37bce634ec0f8b4c43bc9b5e61b3/Circuit_Playground/CircuitPython/CPB_DataLogger/bluefruit_datalogger_MS8607_uart_send.py Raw Data::

Bike test

Need data and format from C

Need Plots

Setup:

# Adafruit Ultimate GPS Breakout - 66 channel w/10 Hz updates - PA1616S

https://www.adafruit.com/product/746

$29.95

## Technical Details

- Satellites: 22 tracking, 66 searching

- Patch Antenna Size: 15mm x 15mm x 4mm

- Update rate: 1 to 10 Hz

- Position Accuracy: < 3 meters (all GPS technology has about 3m accuracy)

- Velocity Accuracy: 0.1 meters/s

- Warm/cold start: 34 seconds

- Acquisition sensitivity: -145 dBm

- Tracking sensitivity: -165 dBm

- Maximum Velocity: 515m/s

- Vin range: 3.0-5.5VDC

- MTK3339 Operating current: 25mA tracking, 20 mA current draw during navigation

- Output: NMEA 0183, 9600 baud default, 3V logic level out, 5V-safe input

- DGPS/WAAS/EGNOS supported

- FCC E911 compliance and AGPS support (Offline mode : EPO valid up to 14 days )

- Up to 210 PRN channels

- Jammer detection and reduction

- Multi-path detection and compensation

Benefits: Location, Time, quality, altitude speed, track angle horizontal dilution, height geoid

SUCCESFULL LOG ON BLUETOOTH UART

Format:

Raw Code:

https://github.com/TheJus10Dyer/Microcontrollers/blob/main/Adafruit%20Bluefruit/GPS_bluefruit.py

Raw Data::

Vehicle Test

Need data and format from C

Need Plots

Need Setup:

LAUNCH